100 n$="0:term.plus.o":open 15,8,15,"s"+n$:close 15:open 1,8,1,n$

110 sys 1000

120 .opt o1

130 ;

140 ; term.plus.64.doc

150 ;

160 ; origion version by frank prindle using the cbm assembler.

170 ; converted by g. r. (fred) walter to assemble using pal.

180 ;

190 ; last date 870102

200 ;

210 ;"This program makes the Commodore 64 operate like a semi-

220 ;"intellegent 40 or 80 column ascii terminal.  It supports input

230 ;"and display of all the ASCII characters, and beeps when the BEL

240 ;"character is received.  Beware that the 80 column display mode

250 ;"will probably require a monitor unless you are using a B/W

260 ;"TV set with outstanding sharpness and video bandwidth.

270 ;

280 ;"The communication protocol is full duplex, 8 bits, no

290 ;"parity, 1 stop bit, 300 or 1200 baud, with XON/XOFF (DC1/

300 ;"DC3) host flow control protocol -- this should satisfy most

310 ;"host computers.  The display screen is 40 columns X 25 lines

320 ;"or 80 columns X 25 lines.

330 ;

340 ;"Because of the unusual layout of the C64 keyboard, the

350 ;"desire to support all ascii characters, and a personal

360 ;"need to find the ESC and BS keys in their usual positions,

370 ;"the following key substitutions apply (note- ^x is short-

380 ;"hand for CONTROL-x):

390 ;

400 ;"       TO SEND         TYPE               really is

410 ;"       -------         ----               ---------

420 ;"       \               BRITISH-POUND      backslash

430 ;"       ^\              ^BRITISH-POUND     ctrl backslash

440 ;"       ^[164]              ^=                 ctrl underscore

450 ;"       ^               UP-ARROW           up-arrow

460 ;"       ^^              ^UP-ARROW          ctrl up-arrow

470 ;"       ^[ (ESC)        ^[ or BACK-ARROW   escape

480 ;"       [164]               f1                 underscore

490 ;"       [C=*]               f2                 pipe

500 ;"       [SHIFT_@]               f3                 backwards '

510 ;"       [175]               f4                 tilde

520 ;"       [SHIFT_+]               f5                 left curly bracket

530 ;"       [SHIFT_-]               f6                 right curly bracket

540 ;"       DEL (RUB)       f7                 delete

550 ;"       ^T              f8                 ctrl t

560 ;"       ^H (BS)         DEL, ^H, or ^T     backspace

570 ;"       ^J (NL)         SHIFT/RETURN or ^J newline

580 ;"       ^S (DC3)        ^S or HOME

590 ;"       ^Q (DC1)        ^Q or CURSOR DOWN

600 ;"       ^I (TAB)        ^I (ie. No separate TAB key)

610 ;

620 ;"The CONTROL-8 key is used to switch the program from 40 column

630 ;"to 80 column mode.  The CONTROL-4 key is used to switch the

640 ;"program back from 80 column to 40 column mode.  Depressing one

650 ;"of these keys when already in the corresponding mode will simply

660 ;"clear the screen (as it does when switching modes).

670 ;

680 ;"Note that pressing the COMMODORE key will place the terminal

690 ;"program in a suspended display mode (ie. the cursor will freeze

700 ;"at it's current position), and ^S will be sent to the host

710 ;"computer when the RS-232 input buffer fills up.  Similarly,

720 ;"pressing the COMMODORE key will remove the terminal from the

730 ;"suspended display mode and send a ^Q to the host computer only

740 ;"if a ^S had previously been sent and the buffer has become nearly

750 ;"empty.  This enables the COMMODORE key to be used like the SCROLL/

760 ;"NO-SCROLL key on DEC terminals.  Suspended display mode is

770 ;"indicated to the user by the cursor blinking much faster than

780 ;"normal, an indication that the COMMODORE key must be pressed to

790 ;"resume displaying the received text.  The program may also send

800 ;"^S and ^Q to the host computer on it's own from time to time if

810 ;"it detects an impending RS-232 buffer overflow condition - this

820 ;"normally will only occur while printing a number of short or blank

830 ;"lines while scrolling the screen up (or down), since 4 to 15 char-

840 ;"acters are received into the RS-232 buffer during the time it

850 ;"takes to scroll the whole screen.  In any case, this automatic

860 ;"flow control is fully synchronized with the flow control

870 ;"necessitated by the use of the COMMODORE key to suspend and resume

880 ;"the display of received text.  Depressing the ^S and ^Q keys (or

890 ;"their substitutes indicated above) unconditionally sends those

900 ;"ASCII characters to the remote host computer without any local

910 ;"effect.

920 ;

930 ;"This terminal program does 'not' support flow control in the

940 ;"opposite direction - receiving ^S from the host computer will not

950 ;"lock the keyboard or have any other effect, nor will ^Q.

960 ;

970 ;"Also note that pressing the RUN/STOP key with SHIFT depres-

980 ;"sed will cleanly terminate the terminal emulator program,

990 ;"and generate the BRK interrupt to send the C64 back

1000 ;"to whoever is servicing those interrupts (BASIC,

1010 ;"SUPERMON64,etc).

1020 ;

1030 ;"All 94 printable ascii characters and the space will

1040 ;"be displayed on the screen as received over the RS-232

1050 ;"channel.  Most of the remaining (control) characters are

1060 ;"not displayed- however the following control characters

1070 ;"and escape sequences have special effects:

1080 ;"

1090 ;"       CHARACTER       EFFECT

1100 ;"       ---------       ------

1110 ;"       ^G (BEL)        beeps the TV

1120 ;"       ^H (BS)         cursor backspaces

1130 ;"       ^I (TAB)        cursor moves right to next tab stop

1140 ;"       ^J (NL)         cursor moves down one line with scrolling

1150 ;"       ^M (CR)         cursor moves to start of line

1160 ;"       ^[A             cursor moves up one line

1170 ;"       ^[B             cursor moves down

1180 ;"       ^[C             cursor moves right one char.

1190 ;"       ^[D             cursor moves left one char.

1200 ;"       ^[H             cursor homes

1210 ;"       ^[I             cursor moves up one line with scrolling

1220 ;"       ^[J             clears from cursor to end of screen

1230 ;"       ^[K             clears from cursor to end of line

1240 ;"       ^[Ylc           cursor moves to line l-$20, col c-$20

1250 ;"       ^[Z             sends DEC VT-52 terminal id string

1260 ;"       ^[n             (lowercase n) turns off reverse video

1270 ;"       ^[o             (lowercase o) turns on reverse video

1280 ;"       ^[^\            initiates downline load

1290 ;

1300 ;"Tab stops are pre-defined and fixed at every 8 columns

1310 ;"beginning with column 0 (ie. 0, 8, 16, 24, ...).

1320 ;

1330 ;"The rest should all be pretty much self explanatory except

1340 ;"for "downline load".  Once the ^[^\ sequence has been re-

1350 ;"ceived, the program expects lines to come in over the RS-

1360 ;"232 channel of the form:

1370 ;

1380 ;"            ;llhhdddddddddd...

1390 ;

1400 ;"where "llhh" is the address for the first byte of data

1410 ;"on the line (in 6502 low/high format), and "dddddd.."

1420 ;"are the data bytes themselves - note that the address

1430 ;"must be 4 hexadecimal digits and each data byte must

1440 ;"be 2 hexadecimal digits.  The load is terminated normally

1450 ;"by reception of the control character ^^, or abnormally

1460 ;"if the user hits a key on the C64 keyboard during the

1470 ;"load.  In the 40 column mode, It is possible to load anywhere

1480 ;"in the address ranges $0800 through $9fff and $c000 through

1490 ;"$cfff since the terminal program itself hides under the BASIC

1500 ;"ROM (from $a000 through $bfff).  80 column mode should not

1510 ;"be used to download into the address range $5c00 through

1520 ;"$7fff because that is where the high resolution video matrix

1530 ;"and screen image reside in that mode.

1540 ;

1550 ;"VAX UNIX users (Berkeley 4.1bsd) will find that the following

1560 ;""termcap" entries effectively describe this terminal program

1570 ;"for use with the terminal dependent UNIX programs (eg. vi,

1580 ;"emacs, more, rogue, etc.):

1590 ;

1600 ;" qc[C=*]c64[C=*]C64[C=*]Commodore 64:co#40:li#25:\

1610 ;"         :up=\EA:do=\EB:cl=\EH\EJ:am:bs:\

1620 ;"         :bw:ho=\EH:nd=\EC:so=\Eo:\

1630 ;"         :se=\En:cm=\EY%+ %+ :\

1640 ;"         :ce=\EK:sr=\EI:pt:

1650 ;

1660 ;" qd[C=*]c64w[C=*]C64W[C=*]Commodore 64 80 Col X 25 Lin:co#80:li#25:\

1670 ;"         :up=\EA:do=\EB:cl=\EH\EJ:bs:\

1680 ;"         :bw:ho=\EH:nd=\EC:so=\Eo:\

1690 ;"         :se=\En:cm=\EY%+ %+ :\

1700 ;"         :ce=\EK:sr=\EI:pt:

1710 ;

1720 ;"VAX VMS users can probably get away with the following (although

1730 ;"some VMS programs [eg. edt] require an auxilliary keypad, and do

1740 ;"not work especially well in 40 column mode):

1750 ;

1760 ;"         set term /dev=vt52

1770 ;"         set term /pag=25

1780 ;"         set term /wid=80  [or 40, as the case may be]

1790 ;

1800 ;"TOPS-20 users can run "emacs" and the like after the following:

1810 ;

1820 ;"         term vt52

1830 ;"         term length 25

1840 ;"         term width 80  [or 40, as the case may be]

1850 ;"         term pause com

1860 ;

1870 ;"A low pitched beep (much lower than that generated by

1880 ;"receipt of the BEL character) indicates an error on the

1890 ;"RS-232 channel (a hit, lost data, etc).

1900 ;

1910 ;"Of course, this program assumes you have been able

1920 ;"to connect a modem (or null modem) to the C64 user

1930 ;"port, which may or may not be trivial!

1940 ;

1950 ;"To run the terminal program, just load it like any BASIC

1960 ;"program and RUN it.  You will be prompted for the baud

1970 ;"rate.  Enter either 3 or 300 for 300 baud, 12 or 1200

1980 ;"for 1200 baud, or just a carriage return for the default

1990 ;"(300 baud).  If you find communications shaky or impos-

2000 ;"sible at 1200 baud, you may have to change the poke in

2010 ;"basic which pokes 0 to poke 8 instead.  If this fails to

2020 ;"help, try tuning the optional baud rate constants in 0a4f

2030 ;"and 0a50 for your system clock as described in ref. guide

2040 ;"page 350 (use poke of 0 into 0a4d in this case).

2050 ;

2060 ;"Once it has been run, the terminal program moves itself

2070 ;"under the BASIC ROM and runs there (although the original

2080 ;"copy remains where it was loaded and can be re-run as long

2090 ;"as it remains intact).  The memory map at run time is:

2100 ;

2110 ;"       $a080-$afff     The program area

2120 ;"       $b000-$b1ff     Unused

2130 ;"       $b200-$b2ff     RS-232 output buffer

2140 ;"       $b300-$b3ff     RS-232 input buffer

2150 ;"       $b400-$b7ff     Video matrix (the 40 column screen)

2160 ;"       $b800-$bfff     Character set for 40 column mode

2170 ;"       $02a7-$02b6     Termination/Restart code

2180 ;

2190 ;"In addition, the following memory areas are used only while the

2200 ;"program is in the 80 column mode:

2210 ;

2220 ;"       $5c00-$5fff     Video matrix (the 80 column color memory)

2230 ;"       $6000-$7fff     Hires screen (the 80 column screen)

2240 ;

2250 ;"Once the terminal program has been terminated (with SHIFT/

2260 ;"RUN/STOP), it may be re-started by re-running the original

2270 ;"BASIC program.  However, if the BASIC program area has been

2280 ;"over-written during downloading, the terminal program can

2290 ;"still be restarted (at the same baud rate as before) with

2300 ;"the BASIC direct command "(686)" ; this command will

2310 ;"restart the terminal program at any time, even after running

2320 ;"other programs, unless some program is run which pokes

2330 ;"into the areas shown in the first map above.

2340 ;

2350 ;"One last warning: this program is not relocatable - do

2360 ;"not change the *length* of the BASIC program or nothing

2370 ;"will work!

2380 ;

2390 ;"Good Luck and Enjoy....

2400 ;"               Frank Prindle

2410 ;

2420 .file 08,"0:term.lables"